KEN model: changes to agriculture, water and desalination investment

Notes for porting the changes to another model version

Repository KEN_model_public. The agriculture and water supply changes are in two commits of 1 October 2026: d70dddc (agricultural sector, batch 1) and 8fbcc2f (water sector and agriculture, batch 2). The figure function plot_agri_trilemma is in da58ec4 (6 October 2026). The water for mitigation (section 6.4) is in 16feef1 (12 October 2026). The command git diff c04e38f 16feef1 -- model/ shows the complete code difference. Document updated on 13 October 2026.

# 1. Summary

Agricultural is now the sum of crops, non-crop output (mainly livestock) and a value-chain increment that responds to the demand of other sectors.

When agricultural output is replaced, intermediate sales, final demand and imports are rebalanced, so the accounting identities hold for the agricultural sector.

Agricultural water use is computed in physical terms and can be capped. The transformation scenario caps it at its 2021 level of 10.8 billion m3.

The diet and food-waste factor of the transformation scenario now also lowers household demand for agricultural goods, not only domestic supply.

Desalination and wastewater investment has a corrected base and, after the groundwater phase-out, a floor that follows expected water demand. Deliveries are bounded by installed capacity.

Water for mitigation (solar power, hydrogen, electrification, carbon capture, tree planting) is now part of the national water balance. Tree planting gets only the treated wastewater that the reuse sector does not deliver to other users, so this wastewater is no longer counted twice.

There are 11 new scenario parameters and 19 new model variables (12 for agriculture and water supply, 7 for mitigation water).

# 2. Files changed

| File | What changed |
| --- | --- |
| model/water_module.py | get_agr_data (new arguments and return values); new functions compute_agr_supply, agr_domestic_sourcing_share, agr_diet_consumption_shares; replace_agr_X (new signature, rebalancing); calculate_water_demand; calculate_aggregate_supply_mix (new argument p); blank self-sufficiency ratios in step2_calculate_production. Section 6.4: total water use with mitigation water, water_use_supply_mix, wwater_collectable, scaled column totals in water_accounting. |
| model/calibration.py | Section 8.2: new call of get_agr_data. New section 8.2.1 with the calibrated inputs of the value-chain coupling. |
| model/investment_module.py | Section 2: investment base, requirement-based floor, two new variables. Uses water_use_supply_mix (section 6.4). |
| model/energy_module.py | Section 6.4: water intensities as module constants, new helpers wastewater_recovery_rate, wastewater_available_for_nexus, nexus_water_other_m3. Wastewater cap of tree planting (two places), part 6 (water for mitigation) and part 7 rewritten. |
| model/energy_investment_module.py | Nature investment follows the planting deployed in the previous period (section 6.4). |
| model/model.py | Import of agr_diet_consumption_shares; diet shares set at the start of each step; replace_agr_X(v, pc, t, p). |
| model/endogenize_input_output_matrix_A.py | replace_agr_X(v, pc, t, p). |
| model/model_classes.py | 19 new variables in ModelVariables (section 8). |
| model/parameters_*.py (4 files) | 11 new parameters, with scenario-specific values. |
| model/plot_water_results.py | WaterResultRunner no longer re-injects agriculture data; new figure function plot_agri_trilemma. Reuse series include the wastewater used for mitigation. resolve_overlaps takes the axis as second argument. |
| model/experiment_plots.py, model/postprocessing_energy.py | Plots only: reports are cut to 2021 to 2060 before plotting, and the nexus figure reads water_use_nexus and electricity_nexus_water_total from the run. |

# 3. Agricultural output

## 3.1 Crop share from the input-output table

Before, total agricultural output was crop output divided by a hard-coded crop share of 0.34. The share is now derived from the base-year input-output table (IOT):

crop_share_2021 = crop_output_2021 / X_agr_2021_IOT

get_agr_data has a new required argument total_output_2021_iot, to be passed as pc.X_[pc.agr_sector], and raises a ValueError without it. It now returns six values (the sixth is crop_share_2021), or seven with return_details=True (a dictionary used by the coupling in section 3.2). The share is held fixed for all years. In the current data the non-crop part is 83% of the sector, so the crop share is about 17%, not 34%.

The calibration call also passes population_growth_rate=parameters.wage_population_growth_adjustment, so that the crop module and the economic core use the same population path. It is zero in the steady-state run.

## 3.2 Three components of output (value-chain coupling)

The new function compute_agr_supply(v, p, pc, t) returns nominal agricultural gross output. It is used when p.agr_valuechain_coupling is true. Otherwise the old path pc.AGR_X[t-1] is used, as in the steady-state run. All three components are in 2021 prices (1000 SAR); k = t - 1 is the index of the calibrated arrays.

x_crop    = pc.crop_X[k]                              (crop module)

x_live    = pc.X_live_2021 * pop_factor[k] * diet_factor[k]

with X_live_2021 = X_agr_2021_IOT - crop_X[0]

D(t)      = sum over j != agr of  A[agr, j] * X_j(t) / p_j(t)

D_ref(t)  = D(t=1) * pop_factor[k]

increment = delta(t) * max(0, D(t) - D_ref(t))

X_agr(t)  = (x_crop + x_live + increment) * p_agr(t) / p_agr(0)

Non-crop output grows with population and the diet factor. It is no longer tied to crop output, so a fodder phase-out does not mechanically cut livestock output.

D is the real intermediate demand of all other sectors for agricultural goods (food processing, food services and so on). Own use is excluded, so there is no simultaneity with agricultural output.

D_ref is anchored to the model's own demand in t=1, so the increment is zero in 2021 and output matches the base-year data.

delta is the domestic sourcing share of demand growth beyond population. It rises linearly from agr_domestic_share_start in t=1 to agr_domestic_share_target in period agr_domestic_share_target_year and is constant afterwards (agr_domestic_sourcing_share). The rest of the demand growth is imported.

The price indexing in the last line applies only if p.agr_price_indexing is true.

## 3.3 Rebalancing after the replacement

replace_agr_X now has the signature (v, pc, t, p) and is called in two places: model.py and endogenize_input_output_matrix_A.py. After setting agricultural output it does three more things:

It recomputes intermediate sales of all sectors: IntS_ = A · X_.

It sets agricultural final demand to the residual: Y_agr = X_agr - IntS_agr.

It absorbs the change in imports: IM_agr -= (Y_agr_new - Y_agr_old). Consumption, government, investment and export demand for agricultural goods stay untouched.

Before this change, only X_agr was overwritten, and X = IntS + Y did not hold for agriculture.

# 4. Agricultural water

The existing water path is kept: crop withdrawal from the crop module plus the calibrated residual, scaled with crop output. It is stored as pc.agr_water_base. The value-chain increment adds water on top:

w_inc  = theta * I_nonfodder[k] + (1 - theta) * (phi_live + f * I_feed)

I_feed = fodder_water_2021 / X_live_2021

if cap > 0:  increment = min(increment, max(0, cap - water_base[k]) / w_inc)

agr_water_total = water_base[k] + w_inc * increment

| Symbol | Code name | Meaning |
| --- | --- | --- |
| theta | p.agr_increment_crop_share | Crop share of the increment (0.124, the 2021 non-fodder crop share of the sector). |
| I_nonfodder | pc.agr_crop_nonfodder_intensity | Water intensity of non-fodder crops by year, m3 per 1000 SAR. Follows the scenario's irrigation technology, greenhouses and crop mix. |
| phi_live | p.agr_livestock_direct_water_intensity | Direct livestock water (drinking, servicing), m3 per 1000 SAR. |
| f | p.agr_increment_feed_domestic | Share of the feed for additional livestock that is grown domestically. |
| I_feed | pc.agr_feed_water_intensity | Fodder water per unit of non-crop output in 2021. |
| cap | p.agr_water_cap | Ceiling on agricultural water in m3; 0 means no ceiling. A binding cap truncates the domestic increment and imports fill the gap. |

In calculate_water_demand, when the coupling is on, the agricultural water intensity is set to v.agr_water_total[t] / v.X_[t][pc.agr_sector]. The rest of the water module then works unchanged.

# 5. Diet factor on the demand side

In the transformation scenario the crop module lowers per-capita food demand linearly from 1.00 in 2021 to 0.75 in 2035. Before, this lowered domestic supply only and showed up as additional imports. With p.agr_diet_demand_side true, agr_diet_consumption_shares(pc, t) scales the agricultural share of household consumption pc.dC by the same factor. The freed budget is spread pro rata over all other goods, so total consumption does not change.

The call sits in run_model_step directly after calc_io_table, before any use of pc.dC. The function keeps the original shares in pc.dC_base. The factor path is written out a second time in get_agr_data and must be kept in line with step1_calculate_consumption.

# 6. Water supply and investment

## 6.1 Investment base

Desalination and wastewater investment grow from last period's value. The base was v.I_[t-1] of the sector, which also contains add-ons such as energy efficiency investment. After a year with zero capacity investment, these small add-ons became the base and investment never recovered. The base is now the sector's own capacity investment, stored in the new variables v.I_desal_cap and v.I_wwater_cap. They are set at the end of section 2 of investment_module.py, before the add-ons. If the base is zero, depreciation is used, as before.

## 6.2 Requirement-based floor after the groundwater phase-out

Once groundwater is phased out, desalination and reuse must supply all demand, so their capital stock has to follow it. For t > 2 and v.water_gw_phased_out[t-1] > 0, investment gets a floor:

expected_demand  = W[t-1] * (W[t-1] / W[t-2])^2     (W = water_use_supply_mix, section 6.4)

share_desal      = pot_desal[t-1] / (pot_desal[t-1] + pot_wwater[t-1])

K_required       = expected_demand * share * p_m3 / (eK * u)

K_next_without_I = (K[t-1] * (1 - dep) + I[t-1]) * (1 - dep)

I[t]             = max(I[t], K_required - K_next_without_I, 0)

This is done for both sectors, with share_desal and 1 - share_desal and the sector's own price per m3, capital productivity, utilisation and depreciation rate (pc.p_desal_m3, pc.eK_desal, pc.u_desal and the wastewater equivalents). Investment in t becomes capacity in t+1.

## 6.3 Supply mix and capacity limit

calculate_aggregate_supply_mix now takes p as fourth argument, passed from water_accounting.

The phase-out is persistent: the new flag v.water_gw_phased_out is set to 1 once groundwater use falls to zero and stays 1.

After the phase-out, deliveries of each source are limited to potential * (1 + p.water_supply_capacity_headroom), with a headroom of 10%. Spare capacity of one source covers a shortfall of the other.

Any remaining shortfall is stored in v.water_supply_gap. If it exceeds 1 million m3, it is met by groundwater, so the water balance stays closed and the gap is visible as a diagnostic.

## 6.4 Water for mitigation

The energy module computes the water that mitigation needs. Before, this water was not part of the national water balance: the energy module added it to water_use_total_national, and the water module, which runs later in the period, overwrote the total. Tree planting was limited by all collectable urban wastewater, although the reuse sector already delivers part of it to other users.

New helpers at the top of energy_module.py. The water intensities are now module constants (WI_SOLAR, WI_HYDROGEN, WI_SGI_BM3_PER_MT and so on), with unchanged values:

wastewater_recovery_rate(t)          = min(0.30 + 0.02 * (t - 1), 0.80)

wastewater_available_for_nexus(v, t) = urban[t-1] * g_urban * recovery_rate(t) - water_use_wwater_tot[t-1] * g_reuse

nexus_water_other_m3(v, pc, t)       = water for solar, wind, hydrogen, electrification, CCS and CCU

urban is household, industry and service water use. g_urban and g_reuse are the growth factors of the last period: both terms are projected one period ahead, because the water module runs after the energy module.

Tree planting (in both scenario branches) and the sources of the mitigation water (part 6):

avail_for_sgi          = max(0, wastewater_available_for_nexus - nexus_water_other_m3)

flow_remove_nature[t]  = min(intended, avail_for_sgi / 0.315e9)          (0.315 billion m3 per Mt CO2)

water_use_nexus[t]     = nexus_water_other_m3 + flow_remove_nature[t] * 0.315e9

water_energy_wwater[t] = min(water_use_nexus[t], wastewater_available_for_nexus)   (0 without mitigation policy)

shortfall              = water_use_nexus[t] - water_energy_wwater[t]:  70% water_energy_desal, 30% water_energy_gw

In the water module (calculate_water_demand):

water_use_total_national[t] = sectors + households + water_use_nexus[t]

water_use_supply_mix[t]     = water_use_total_national[t] - water_energy_wwater[t]

wwater_collectable[t]       = (households + industry + services) * wastewater_recovery_rate(t)

calculate_aggregate_supply_mix and section 2 of the investment module now use water_use_supply_mix where they used water_use_total_national. Desalination, sector reuse and groundwater supply this amount. The wastewater for mitigation is a supply stream of its own.

In water_accounting the column totals of the RAS matrix are scaled to the demand of the four user groups, because mitigation water is not a row of the matrix.

In energy_investment_module.py the nature investment of the net zero branch follows the planting deployed in the previous period (flow_remove_nature[t-1] - flow_remove_nature[t-2]), not the uncapped target path.

The electricity to supply this water is stored in electricity_nexus_water_total. It is added to electricity generation only with p.nexus_water_electricity_feedback = True. The parameter is read with getattr, is not in the parameter files and is off by default: the load is below 0.4% of electricity use, but it shifts a threshold in the Vision 2030 run around 2040 and moves its 2060 GDP by about 1%.

# 7. New parameters and scenario values

| Parameter | Baseline | Vision 2030 | Transformation | Steady state |
| --- | --- | --- | --- | --- |
| agr_valuechain_coupling | True | True | True | False |
| agr_domestic_share_start | 0.0 | 0.76 | 0.76 | 0.76 |
| agr_domestic_share_target | 0.0 | 0.90 | 0.85 | 0.76 |
| agr_domestic_share_target_year | 1 | 10 (2030) | 15 (2035) | 1 |
| agr_increment_crop_share | 0.124 | 0.124 | 0.124 | 0.124 |
| agr_increment_feed_domestic | 1.0 | 0.3 | 0.0 | 1.0 |
| agr_livestock_direct_water_intensity | 3.0 | 3.0 | 3.0 | 3.0 |
| agr_water_cap | 0 | 0 | 10.8e9 | 0 |
| agr_diet_demand_side | False | False | True | False |
| agr_price_indexing | True | True | True | False |
| water_supply_capacity_headroom | 0.10 | 0.10 | 0.10 | 0.10 |

How to read the scenario settings: in the baseline, demand growth beyond population is imported, which keeps the baseline on its calibrated trend. Vision 2030 localises 90% of that growth by 2030 with no water ceiling. The transformation scenario localises 85% by 2035 within the water ceiling, with additional livestock on imported feed. The value 0.76 is the 2021 domestic supply share of agricultural goods, X / (X + IM).

# 8. New model variables

| Variable | Content |
| --- | --- |
| agr_X_crop | Crop output from the crop module (1000 SAR, 2021 prices). |
| agr_X_livestock | Non-crop output, growing with population and diet factor. |
| agr_X_valuechain | Domestic value-chain increment. |
| agr_D_valuechain | Real intermediate demand of all other sectors for agricultural goods. |
| agr_domestic_share_delta | Domestic sourcing share of the period. |
| agr_water_valuechain | Water use of the increment (m3). |
| agr_water_total | Total agricultural water use including the increment (m3). |
| agr_water_cap_slack | Cap minus agricultural water use (m3); 0 if there is no cap. |
| I_desal_cap | Own capacity investment in desalination, before add-ons (1000 SAR). |
| I_wwater_cap | Own capacity investment in wastewater treatment, before add-ons (1000 SAR). |
| water_supply_gap | Demand that desalination and reuse capacity cannot cover after the phase-out (m3). |
| water_gw_phased_out | 1 once groundwater abstraction has been phased out, else 0. |
| water_use_nexus | Water use of mitigation technologies and tree planting (m3), part of water_use_total_national. |
| water_energy_wwater | Part of it supplied by reclaimed wastewater not delivered to other users (m3). |
| water_energy_desal | Part of it supplied by desalination (m3), shortfall only. |
| water_energy_gw | Part of it supplied by groundwater (m3), shortfall only. |
| water_use_supply_mix | Water use met by desalination, sector reuse and groundwater: total minus water_energy_wwater (m3). |
| wwater_collectable | Collectable urban wastewater: recovery rate times household, industry and service water use (m3). |
| electricity_nexus_water_total | Electricity to supply the water for mitigation (GJ). |

Section 8.2.1 of calibration.py adds the calibrated inputs: X_live_2021, agr_pop_factor, agr_diet_factor, agr_water_base, agr_crop_nonfodder_intensity, agr_feed_water_intensity, agr_domestic_share_2021, crop_share_2021 and the diagnostic D_agr_2021.

# 9. Smaller changes

Blank self-sufficiency ratios (for example wheat after 2021 in the Vision 2030 and transformation input files) are now set to zero explicitly in step2_calculate_production. Results do not change, because these rows were already dropped from the yearly sums.

WaterResultRunner in plot_water_results.py no longer calls get_agr_data or overwrites pc.crop_X, pc.AGR_X and the water intensity. These are set once in the calibration; the runner only reads the irrigation cost series from pc.

New figure function plot_agri_trilemma(all_results, all_sectoral_results) in plot_water_results.py, called from plot_water_results(): domestic agricultural output, agricultural imports and agricultural water use by scenario.

plot_agri_trilemma shows output and imports in current prices (X, IM), like the other monetary figures. The check values in section 11 are in 2021 prices (x, im).

experiment_plots.py cuts all reports to 2021 to 2060 before plotting (_truncate_reports). The 2021 start labels of the GDP shares are computed from the Baseline run and no longer hard-coded.

# 10. Suggested order for porting

Add the 11 parameters to all parameter files and the 12 variables to ModelVariables.

Update get_agr_data and its call in the calibration, and add section 8.2.1.

Add agr_domestic_sourcing_share, compute_agr_supply and agr_diet_consumption_shares, and replace replace_agr_X.

Update both calls of replace_agr_X and add the diet-share call in run_model_step.

Update calculate_water_demand and calculate_aggregate_supply_mix, and pass p in water_accounting.

Update section 2 of the investment module.

Add the helpers, the wastewater cap and the new part 6 in energy_module.py, the 7 variables of section 6.4, and the water_use_supply_mix changes in water_module.py and investment_module.py.

Remove the agriculture injection from any custom runner, as in WaterResultRunner.

# 11. Values to check after porting

With the current code (commit 16feef1, run of 13 October 2026), the 2060 values are:

| 2060 | Baseline | Vision 2030 | Transformation |
| --- | --- | --- | --- |
| Agricultural water use, billion m3 | 16.8 | 17.0 | 10.8 |
| Domestic agricultural output, bn SAR (2021 prices) | 213.5 | 383.3 | 242.5 |
| Agricultural imports, bn SAR (2021 prices) | 246.4 | 296.8 | 354.3 |
| Total water use, billion m3 | 28.9 | 32.7 | 29.1 |
| Water for mitigation, billion m3 (all from treated wastewater) | 1.20 | 1.27 | 3.08 |
| Sector reuse plus mitigation wastewater / collectable wastewater, billion m3 | 5.55 / 8.68 | 7.88 / 11.58 | 12.15 / 12.17 |
| Tree planting, Mt CO2 per year | 3.6 | 3.6 | 8.6 |

In 2021 the increment must be zero and agricultural water use must be 10.8 billion m3 in all scenarios. Sector reuse plus mitigation wastewater should stay within wwater_collectable (see section 12 for the exception). In the transformation scenario the wastewater limit holds tree planting to 8.6 Mt in 2060.

# 12. Open points

agr_livestock_direct_water_intensity = 3.0 m3 per 1000 SAR is a placeholder, to be verified with herd and water data from the ministry (MEWA).

The diet factor path (1.00 to 0.75 by 2035) exists in two places, see section 5.

If the water module fails to load in the calibration, the fallback sets the old arrays to zero but not the new inputs of section 8.2.1. A run with the coupling switched on would then stop with a missing attribute.

The wastewater available for mitigation is projected from t-1. If water use jumps from one year to the next, sector reuse plus mitigation wastewater can slightly exceed wwater_collectable in that year.

The Vision 2030 run is sensitive to small changes in electricity demand (section 6.4). After porting, compare all scenarios with saved results.